# Practical 7

Objective: NAT (Network Address Translation): Set up NAT on a router to translate private IP addresses to public IP addresses for outbound internet connectivity. Test the translation and examine how NAT helps conserve IPv4 address space.(Using Packet Tracer) Procedure: Step 1: Set Up the Devices

- Open packet Tracer.

- Add the following Devices to the workspace:

    - 2 Router

    - 2 PC

    - 2 Switch

    - 1 Server

- Connect Devices:

  - Use straight-through ethernet cable to link the devices together (PCs, Switch, Server, router).

  - For connection of router0-to-router1 communication, use serial DTE wire(serial2/0 to serial2/0).

Router(config)#interface fastethernet 0/0

Router(config-if)#ip nat inside

Router(config-if)#exit

Router(config)#interface serial 2/0

Router(config-if)#ip nat outside

Router(config-if)#exit

Router(config)#

Router(config)#interface Serial2/0

Router(config-if)#

Router(config-if)#exit

Router(config)#

Router(config)#ip route 60.0.0.0 255.0.0.0 192.162.10.2 Router(config)#exit

4. Assign IP address to router1 in Config.

- Go in Config then • FastEthernet 0/0- Port Status: on IPv4

Address: 20.20.20.1

Subnet Mask: 255.0.0.0 •

Serial2/0- Port Status: on

IPv4 Address: 192.162.10.2

Subnet Mask: 255.255.255.0

- CLI command in router type following ccommands  Router>enable

Router#

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#exit

Router(config)#ip nat inside source static 20.20.20.2 60.60.60.2

Router(config)#interface fastethernet 0/0

Router(config-if)#ip nat inside

Router(config-if)#exit

Router(config)#interface serial 2/0

Router(config-if)#ip nat outside

Router(config-if)#exit

Router(config)#ip route 50.0.0.0 255.0.0.0 192.162.10.1 Router(config)#exit

- Configure the Server

  - Click on Server0

  - Go to the Services tab

  - Click on HTTP.

  - Ensure the HTTP services is ON.

  - Edit text in index.html

- Test the translation

1) Verify pass message PC0 to PC1

# Practical 8

Objective: Network Troubleshooting- IP Misconfiguration: Identify and resolve network connectivity issues caused by an incorrect IP configuration. We will create a simple network in Cisco Packet Tracer (2 PCs, 2 routers, 2 switches) where one device’s IP settings are intentionally wrong, then use simulation mode and CLI troubleshooting to find and fix the error.

## Procedure

- Step 1: Set Up the Devices o Open Cisco Packet Tracer. Add the following devices to the workspace:

    - 2 Routers (e.g. Router0, Router1)

    - 2 Switches (Switch0, Switch1)

    - 2 PCs (PC0, PC1)

  - Connect devices with straight-through cables as follows:

    - PC0 → Switch0, and PC1 → Switch1.

    - Router0 (FastEthernet0/0) → Switch0; Router1 (FastEthernet0/0) → Switch1.

  - Connect the routers to each other: Use a serial DTE cable to link Router0’s Serial2/0 port to Router1’s Serial2/0 port (one end will be DCE).

- Step 2: Assign IP Addresses (with an intentional error) o PC

Configurations:

      - PC0: Go to PC0 → Desktop tab → IP Configuration. Set:

      - IPv4 Address: 192.168.1.10

      - Subnet Mask: 255.255.255.0

      - Default Gateway: 192.168.1.1 (Router0’s Fa0/0)

      - PC1: Go to PC1 → Desktop → IP Configuration. Set:

      - IPv4 Address: 192.168.2.10

      - Subnet Mask: 255.255.255.0

      - Default Gateway: 192.168.1.1 (Incorrect – should be 192.168.2.1)

Note: This mismatch of the gateway on PC1 is the intended misconfiguration.

  - Router0 Configuration (via CLI):

Click Router0 → CLI tab. Enter:

Router0> enable

Router0# configure terminal

Router0(config)# interface FastEthernet0/0

Router0(config-if)# ip address 192.168.1.1 255.255.255.0

Router0(config-if)# no shutdown

Router0(config-if)# exit

Router0(config)# interface Serial2/0

Router0(config-if)# ip address 10.0.0.1 255.255.255.252

Router0(config-if)# no shutdown

Router0(config-if)# exit

Router0(config)# ip route 192.168.2.0 255.255.255.0 10.0.0.2 Router0(config)# exit

o Router1 Configuration (via CLI):

Click Router1 → CLI tab. Enter:

Router1> enable

Router1# configure terminal

Router1(config)# interface FastEthernet0/0

Router1(config-if)# ip address 192.168.2.1 255.255.255.0

Router1(config-if)# no shutdown

Router1(config-if)# exit

Router1(config)# interface Serial2/0

Router1(config-if)# ip address 10.0.0.2 255.255.255.252

Router1(config-if)# no shutdown

Router1(config-if)# exit

Router1(config)# ip route 192.168.1.0 255.255.255.0 10.0.0.1 Router1(config)# exit

- Step 3: Test Connectivity (Initial Ping - Should Fail) o On PC0, go to Desktop → Command Prompt. Issue the ping: o PC> ping 192.168.2.10

  - Switch Packet Tracer to Simulation mode (button at bottom right). Clear filters except ICMP so we can trace the ping. Then send the ping again from PC0.

  - In the Event List, observe the following:

      - PC0 ARPs for 192.168.1.1 (its gateway) and sends the ICMP echo to Router0.

      - Router0 forwards to Router1, and Router1 ARPs for PC1. PC1 receives the ICMP request.

      - Critical: PC1 then issues an ARP for 192.168.5.1 (its configured gateway) with no reply. This shows PC1 is trying to reach the wrong gateway (192.168.5.1 instead of 192.168.2.1).

- Step 4: Troubleshoot the Misconfiguration o On PC1, check the IP configuration. In Desktop

> Command Prompt, run: o  PC> ipconfig

or simply look at the IP Configuration settings. You will see: Default Gateway = 192.168.5.1 (incorrect).

    - Alternatively, on Router1 CLI, use show ip interface brief to verify Router1’s Fa0/0 address is 192.168.2.1, confirming the gateway should be that.

    - Conclusion: PC1’s default gateway is wrong. It should be the address of Router1’s interface on that network.

- Step 5: Correct the Configuration

    - Fix PC1’s gateway: Click PC1 → Desktop → IP Configuration. Change Default Gateway to 192.168.2.1 (Router1’s Fa0/0 address).

    - Ensure all other settings remain: IPv4=192.168.2.10, Mask=255.255.255.0. Save/close the settings.

- Step 6: Verify Connectivity (Ping - Should Succeed) On PC0, in Command Prompt, run the ping again: o PC> ping 192.168.2.10

This confirms PC0 can reach PC1.

- (Optional) On PC1, try ping 192.168.1.10 to verify two-way communication.

- All pings should now succeed, indicating the network is fully connected after correcting the IP misconfiguration.

# Practical 9

Objective: To monitor network traffic using Wire Shark.

Procedure:

Step 1: Open Wireshark.

- Right-click on Wireshark icon → Run as Administrator.

Step 2: Select the appropriate network interface (such as Wi-Fi or Ethernet) from the list shown.

- Choose the one that shows active traffic (moving graph).

Step 3: Start capturing packets.

- Click the blue shark fin icon to start live capture.

Step 4: Perform some network activity.

- Open a web browser and visit any website (e.g., www.google.com) to generate traffic.

Step 5: Observe live packet capture.

- Packets will appear in the capture window with details like Source, Destination, Protocol, and Info.

Step 6: Stop the capture.

- After a few seconds, click the red square (Stop) button to end capturing.

Step 7: Analyze the captured packets.

Click on any packet to expand and view detailed header information of protocols like:

- Ethernet (Data Link Layer)

- IP (Network Layer)

- TCP/UDP (Transport Layer)

- Application Layer protocols (like HTTP, DNS, etc.)  Practical 10

Objective: To analyze complete TCP/IP protocol suite layer’s headers using Wire Shark.

Procedure:

Step 1: Open Wireshark.

- Right-click on Wireshark icon → Run as Administrator.

Step 2: Select the appropriate network interface (such as Wi-Fi or Ethernet) from the list shown.

- Choose the one that shows active traffic (moving graph).

Step 3: Start capturing packets.

- Click the blue shark fin 영옂옃엿였 icon to start live capture.

Step 4: Run Some TCP Traffic

- Open a web browser (Chrome, Firefox, Edge).

- Visit a website like https://www.google.com or https://www.wikipedia.org. (Websites use TCP).

Step 5: Observe live packet capture.

- Packets will appear in the capture window with details like Source, Destination, Protocol, and Info.

Step 6: Stop the capture.

- After a few seconds, click the red square (Stop) button to end capturing.

Step 7: Select a TCP/IP packet from the captured packets list.

- Identify a packet using TCP or UDP as the transport protocol.

Step 8: Expand and examine each layer.

- Click on the packet and expand the following protocol layers:

  - Ethernet II (Data Link Layer) → Source and Destination MAC addresses.

  - Internet Protocol (IP) (Network Layer) → Source IP, Destination IP, TTL, Protocol, etc.

  - Transmission Control Protocol (TCP) or User Datagram Protocol (UDP) (Transport Layer) → Ports, Sequence numbers, Flags.

  - Application Layer (e.g., HTTP, DNS) → Application data and methods.

Step 9: Note down the header fields.

- Observe how each layer adds its own header information during packet transmission